Arduino Code

#include <Wire.h>

#include <LiquidCrystal_I2C.h>

// Define LCD display (16x2 with I2C)

LiquidCrystal_I2C lcd(0x27, 16, 2);

// Define pins

#define gasSensor A0  // MQ-5 Gas Sensor on A0

#define relayPin 7     // Relay for solenoid valve

#define buzzer 8       // Buzzer alarm

#define redLED 9       // Red LED (Warning)

#define greenLED 10    // Green LED (Safe)

// Gas sensor threshold

#define GAS_THRESHOLD 400

void setup() {

pinMode(gasSensor, INPUT);

pinMode(relayPin, OUTPUT);

pinMode(buzzer, OUTPUT);

pinMode(redLED, OUTPUT);

pinMode(greenLED, OUTPUT);

digitalWrite(relayPin, HIGH); // Keep gas valve ON initially

digitalWrite(buzzer, LOW);

digitalWrite(redLED, LOW);

digitalWrite(greenLED, HIGH); // Green LED ON (Safe)

Serial.begin(9600);  // Serial monitor

lcd.begin();

lcd.backlight();

lcd.setCursor(0, 0);

lcd.print("Gas Detector ON");

delay(2000);

lcd.clear();

}

void loop() {

int gasValue = analogRead(gasSensor);

Serial.print("Gas Sensor Value: ");

Serial.println(gasValue);

lcd.setCursor(0, 0);

lcd.print("Gas Level: ");

lcd.print(gasValue);

if (gasValue > GAS_THRESHOLD) {

Serial.println("Gas Leak Detected! Shutting off gas supply.");

digitalWrite(relayPin, LOW);  // Turn OFF gas valve

digitalWrite(buzzer, HIGH);   // Turn ON buzzer

digitalWrite(redLED, HIGH);   // Red LED ON

digitalWrite(greenLED, LOW);  // Green LED OFF

lcd.setCursor(0, 1);

lcd.print("WARNING! LEAK!!");

delay(5000);  // Alarm duration

}

else {

digitalWrite(relayPin, HIGH); // Keep gas valve ON

digitalWrite(buzzer, LOW);    // Turn OFF buzzer

digitalWrite(redLED, LOW);    // Red LED OFF

digitalWrite(greenLED, HIGH); // Green LED ON

lcd.setCursor(0, 1);

lcd.print("Status: SAFE    ");

}

delay(2000);  // Wait before next reading

}